Transcript Mariusz Zalencki								26.01.27

Meeting Context

Discovery interview with Marius, a development engineer at an automotive supplier (Autoneum), conducted by René, Olaf, and Moës who are exploring software opportunities in electromechanical product engineering.

Marius's Background

- Works in development with focus on engine parts and learning underbody topics

- Manages team of three engineers in Poland

- Company specializes in acoustic components for automotive OEMs (BMW, Mercedes, etc.)

Development Process Overview

- RFQ Phase: Quote preparation phase where company tries to win business based on cost

- Development Phase: Starts after nomination, runs from Gate 1 to Gate 5 (SOP) plus 3 months post-SOP

- Timeline: Typically 2-3 years, but increasing pressure to deliver faster

- Fast-track approach: "Checklist" method used when speed is critical - focuses on key deliverables rather than full structured program

Key Development Work Streams

- CAD/Geometry work: Freezing part design and dimensional specifications

- Requirements analysis: Working with customer spec books (50-80 pages) containing norms and requirements

- Bill of Materials (BOM): Material selection based on performance requirements

- Functional analysis: Defining what performance the product must deliver

Critical Requirements Examples

- Flammability: Legal requirement that parts cannot burn (cascaded from government to OEM to supplier)

- Media resistance: Parts in engine compartment must resist engine oil, coolant, windshield wiper fluid, brake fluid

- Acoustics: Primary function for Autoneum's products

DFMEA (Design Failure Mode and Effects Analysis)

- Purpose: Risk analysis tool used during development to assess design risks

- Current tools: Company uses Plato software (previously Siemens Q-SYS); Plato is more user-friendly

- Process: Takes functional analysis requirements as input and performs systematic risk assessment

- Three key ratings:

  - Severity: How serious is the failure effect (1-10 scale, from no effect to health/safety risk)

  - Occurrence: How likely is the failure to happen

  - Detection: How well can the failure be detected

- Risk calculation: Software multiplies Severity × Occurrence × Detection to determine risk level (High/Medium/Low) based on IATF guidelines

- Special/Critical Characteristics: Requirements related to health, safety, or legal compliance receive highest severity ratings

Part Cost Calculation (PCC)

- Standardized Excel-based process for calculating part costs

- Components included:

  - Bill of materials with material costs per unit

  - Process costs (cycle time, labor rates, operators)

  - Fixed costs (energy, scrap rate, equipment usage)

  - Manufacturing location comparison (multiple plants with different cost structures)

  - Logistics costs (raw material delivery and finished goods shipping)

- Scenario planning: Typically create 2-3 cost scenarios for customer with different requirement levels

- Challenge: Creating multiple scenarios for multiple parts (e.g., 5 scenarios × 10 parts = 50 variants) is time-consuming

Key Pain Points Identified

- Material maturity: Biggest challenge is uncertainty about new materials and material combinations

  - "Maturity" = level of certainty/documented evidence about material performance

  - Higher maturity = lower development costs (fewer tests needed)

  - Lower maturity = higher costs and longer timelines

- Testing time and costs: Some tests take 6 weeks (e.g., climatic chamber aging tests) vs. 5 minutes (e.g., flammability)

  - Without documented evidence from past tests, must perform expensive new tests

  - Creates risk and uncertainty during RFQ phase

- Speed pressure: Customers demanding faster development cycles

  - Need to prepare multiple scenarios quickly

  - Difficult when working with low-maturity materials/designs

- Knowledge silos: Multiple engineers doing similar work without knowing what others have tested

  - No centralized way to access past test results and learnings

  - Duplicated effort and missed opportunities to leverage existing knowledge

- Manual processes: Both DFMEA and cost calculations currently done manually

Pre-Development Phase

- Purpose: Analyze future customer requirements 2-3 years ahead

- Activities: Research market trends, predict product evolution (e.g., combustion engine to battery electric)

- Challenge: Managing transition from pre-development to development - determining what analysis to carry forward

- Opportunity: Software should support multiple scenarios that can evolve as knowledge grows

Need for Agility and Flexibility

- Modern development requires ability to switch between scenarios quickly

- Early in development: may have 5 scenarios

- As knowledge grows: can eliminate some scenarios or combine concepts into new solutions

- Overly structured approaches limit ability to pivot

AI and Automation Opportunities

- Current direction: Company connecting Plato with AI-powered design engagement tools

- Vision: Rather than starting from scratch, AI analyzes past work in database and generates starting point for review

- Tool mentioned: Design engagement tools (possibly from company providing "Colab" or similar - not Google Colab)

- Potential applications:

  - Analyze database of previous part cost calculations to suggest lowest-cost solutions

  - DFMEA generation based on past similar projects

  - Prediction/simulation to eliminate unsuitable materials early

  - Help individual engineers access institutional knowledge

Cross-Functional Collaboration

- Development involves teams ranging from 3-5 people to 100+ people working simultaneously

- Software solution needs to support multiple users collaborating on same topics

Documentation Requirements

- IATF automotive standards require documented evidence for all decisions

- High stakes: failures can result in market returns and significant financial liability

- Cannot rely solely on predictions without test evidence

Next Steps

- Marius open to future follow-up conversations if needed

- Team to consider insights on DFMEA methodology, material maturity challenges, and AI automation opportunities

Notizen

Transkription

But the underbody topics are quite... Fresh to me. So I insert the learning curve. but okay the the engine parts i know quite good and yeah this is where i am i have a team of engineers in poland three people before it was uh Now we change the setup a bit. So that's quickly about me.

Thank you very much for the short introduction. I make a short introduction of all of us because if we go around the table we're just losing time. So we're René, Olaf and Moës and we are exploring a little bit the idea of to develop a software in the engineering, product engineering world, electromechanical parts and components. And we're just trying to find out what are the pain points, what's working well in the process and so on.

Electromechanical.

Yeah, of any, so electro or mechanical parts, and automotive supply, machinery, yeah, all those parts.

Right. What the software is to support the development of parts, right?

Yes, yes.

So you want to support from... From the start to the end, the whole process?

We don't know yet. We are trying to discover what's the pain points in the process, so what's really not working well. To understand a little bit, we already found out a little bit that the trigger for a new development process, especially as a supplier like Autonium, is new requirements from the customer, right? Normally, you start the process when the customer is contacting you, no?

Yes, we have. So I'm in the development. Before we have so-called RFQ. phase where we try to get the business and Basically, we quote toward the customer and try to win. Yeah, of course, money plays the role. So cost, part cost production is the main factor there. And when we win the business, we are getting nominated. And this is the moment where we switch from the other queue. and the development.

And in Autonet, specifically, this development phase is until a great five plus three months. So we have gauges in AutonetLum from one to five. And after this fifth gate, Fifth is SOP and three months after SOP. So basically from the start to the serial production plus three months. and now to whom we are doing the development phase.

And how long does it normally take, this entire phase you just described?

Usually two to three years. It was. But now I see, I observe that the trend is to be faster and faster. Mm-hmm. So basically sometimes we are going outside of standard program trace and we are doing something with so-called checklist means we need to do something really fast. We go more not so structured and not so many Because in the development at some point you need to deliver, we call it deliverables.

So it means that you freeze the design, you freeze what you will test on the part. you freeze what you will test on the material and in a company like Autonome it's quite structured, big files behind and a lot of work and sometimes if we need to work really fast for example customer says he wants to have first of two parts in three months We do it more quick and dirty and then it's called not a program but a checklist.

So the PM who is managing this is just picking the key points which needs to be delivered in the program. to do it faster and not to do full package that is You know, from... In Automotive World you have IETF standard which requires you to have a lot of things done, right? Formally. This checklist is then a faster way to do it. Yeah, so then we focus on freezing the The part design from geometrical point of view, so all this work with the CAD data.

This is the First tasks and then the second one is customer requirements. So the so-called functional analysis, right? So you can imagine you have product A and needs to have specific geometry dimension ABC or XYZ. And then you need to specify what is the bill of material. And so this is the material and the geometry. But then you also need to fulfill what this product has to bring to the customer.

What is the performance of this product expected by the customer? So in our world, Autonom is acoustic. is the main driver that's why we have That's why our customers are buying those parts, but besides the acoustics, This is the main function, acoustic. But there are also functions like flammability, so every part which is installed in the car cannot burn. Hmm. because this is legal requirement coming from the government.

And then our customers like the BMW, Mercedes and all those on the market, they are required by the law that those parts are not burning. So they give us they cascade down from the government, it's being cascaded to the customer and then customer is cascading this requirement to us. And the part doesn't have to be burned. This is one example. this flammability what you need to fulfill for the for the part so basically I will try to keep it simple for you and just to have this one example, but normally you have a spec book of sometimes 50, 80 pages.

with all the norms. and customer norms which are specifying specific requirement. I can just give you a few examples. For example, this is flammability, for example, media resistance. You can imagine that the part is assembled in the engine compartment. You basically open the hood of your car, there is the engine and behind it, there is our part. And you can imagine that this part cannot absorb, for example, liquids like engine oil Aure.

coolant or windshield wiper or brake fluid or this kind of. So this will be another requirement next to flammability. And then you get the list of requirements. You tried in the spec book? There are many norms behind as well that you need to fulfill and this is something that you really have to consider. or your product development.

Yeah, and next to the spec book you're also getting the shape of form in the form of a cut. Exactly, exactly.

Yes, so if you imagine you can divide the work into two groups. One is like working with the cut and the geometry And then working with the requirement because the If you analyze the requirements, then this will give you output of the requirement analysis will give you what will be the BOM, the bill of material for this, right? Because you can imagine that Some materials will give you the performance that you need and some of them you...

they will not provide it. So you need to think what materials to use and so on. I think this is-This is mainly the development process that you get the requirements, you analyze what you can meet. and you identify what you cannot meet.

The interesting part is how do you analyze it? How do you get from the spec book of 80 pages to a bomb of some materials, some components?

Do you know a tool which is called DFMEA?

D-F-M-E-A, no? I only know Kranta.

Design failure mode risk analysis. So I would say for the for if you want to create a software For developing the products, this is one of the biggest parts There are already softwares you can check like Plato. You know?

Yeah, Plato I know.

So for example, this is we use software plateau before we used from Siemens Q-SYS. And if I compare those two, the plateau is much more user-friendly. It's, you know, yeah, just much more, you are much more, you can be much more fluent and faster using it. Uh-huh. So this is one of the tools that you... You create your functional analysis. So basically it could be a simple, I don't know, Excel file or whatever that you put the list, what you need to fulfill.

So basically you read the spec book and you try to list what what your part has to fulfill, right? When you have this list, you This is your input. to the DFMEA. Mm-hmm. And in this DFMEA design failure mode risk analysis, It's... you analyze I would need to show you the example.

No, no, no, I know it.

to explain. Basically, there is a IATF handbook, this probably I can show to you, but okay. I don't know if this will be maybe going too much in the details, but basically what is important, you have your requirements in this functional analysis. One of the most important thing coming from the customer as a requirement, is what we call special and critical characteristics. For example, this flammability, It's a critical characteristic.

Why it's critical? Because it's related to the health and it is the health of the final user of the car. Because if the part will burn, the car can burn and then the final user can get harmed. And it's also critical because it's a legal requirement. So whatever is a legal requirement is And this analyzes treated as a critical characteristic And this that you group this specific requirement as a critical gives you then Yeah.

You need to apply specific rating in this design. I will try to show you maybe Okay. Okay. I'm not sure if I'm allowed to do it, you know, this is a bit difficult part for me what I can disclose and what I cannot, right? Maybe I will try to look for the IATF spec book, which is official document available on the market. And then for sure this I can show to you. Maybe you can get it for a couple of euro probably.

I will show my screen. Let me know if you can see. Yes. Okay, so this is FMEA Handbook, right? Failure mode and ethical analysis. You can see here that there is a design FMEA and process FMEA. Mm-hmm. In the development phase, we are basically focusing on the design FMEA. because we are focused on the design of the part. And this you can see is 240 pages and it's created by IAC and VDA before. Maybe you know VDA before.

Now they joined with IAC and this is common work of let's say German and American car manufacturers, right, together. And this whole document gives you guides you or gives you a framework how to perform your design FMEA, so this risk analysis. This is the tool to basically to analyze the risk during your design process. Basically, you design this You design your part, you say, okay, my one of the requirements is flammability.

I will stick to this example to make it simple. And then you say, okay, What I have to meet the customer gives me that my flame ability rates on the speed that the material is burning is below 100 millimeter per minute, for example. And then you say, okay, my failure mode is that something is going wrong. So what if... What is the opposite ofAll right. that I meet this requirement. The opposite is that the part will burn, okay?

So you analyze, okay, and then so if the part will burn, What is the effect on the final user? So you say, okay, the car will get on fire. There is a passenger injury risk. And this is... Then you need to write it. Ah. I wish I do. Find if there is a specific table maybe.

The question then I have from the next one is how do you make a trade-off with costs for example because you're running through all this process to then make an offer right to then say to the company to BMW or Mercedes of hey we can produce this part however it will cost that much. Yes. Is this all done manually or is there a solution in place or...

No, we have a standardized document called PCC, Part Cost Calculation. So basically, it's an Excel file where you put together your bill of material your process, so you defined what is the cycle time. How many operators are there? Mm-hmm. And This is giving you the labor rate behind to produce this part? And you also put together in the bill of material you have to list your materials. For example, you have five materials and you say material A costs one euro per one square meter.

Material B is two euro per square meter, material C is three euro per square meter. You put it together and then you say, okay, A material to produce this part will cost 3 euro, my labor work is, I don't know, 1 euro. Then there's some let's say fixed costs like energy, so you decide in which manufacturing plant you will make it. And you say, okay, I will make it in plant, number one in Europe. The energy cost is this, the scrap rate in this plant is this, the OE, so the usage of the equipment in this plant is this.

My fixed costs are No. additional one euro, right? And then you compare it to another plant where you can manufacture the same part in a company like Autonome, you have more plants. So you say, okay, I need to decide if I want to make it in plant A or B or C. So then you compare the logistic costs because you need to deliver to specific location on the customer side and you have those plant rates which are different.

So you can imagine that salary costs are higher in Switzerland than in Poland, for example. So you compare all those, you put it together with the logistic cost because then you need to deliver your raw materials from Imagine you have your supplier in Germany and you have your production plant in Germany, so it's closer and so on, right? Then to deliver to Poland. So you need to find the best balance between There's logistic cost, production cost, It's a parallel, right?

And you sum up all all this together. and you say, okay, my part production cost is 10 Euro. Mm-hmm. But if, dear customer, you want me to fulfill Another requirement, for example, needs to meet higher flammability level or needs to resist to another media, bigger group of media. That maybe I need to switch my material in the bill of material from A to material B, which costs more. So then you will say, okay, Your product, your customer in version A will cost you 10 euro, in version B with higher requirement 12 euro or so on.

You can leave options to the customer.

So you normally make one or two, two, three scenarios.

Yes. For the customer.

And what is one of the pain points that is not working in this process? So where do you feel sometimes it's stuck?

It's called the maturity of the product or the material that you want to use. So means maturity, I would call it, it's a level of certainty. That you are certain of this specific material performance. So it means how much you know the material. How many tests you have realized in the past that you know that this material with the combination with material A with the combination with material B is giving you this level of performance for this customer test.

Mm-hmm. So you need to find out, okay, there are some, for example, aging tests, which are... Longing six weeks, for example, right? Because okay, flammability, the test you can do in five minutes and you have the result. But the most challenging part is when you have some tests that are taking six weeks, for example, to find climatic chamber you put the material for example into the clatic chamber and you So you assess the performance the bill of material or the park in the long run you can imagine that that costs money so it's not something that you can do really quick it's money so these These are the The challenge is When you don't have Documents would be called an auto ...business documented evidence for means Specific test results available.

We have test reports from the laboratory that says, okay, your product after four weeks in the climatic chamber and temperature of, I don't know, 200 degrees. With this specific bill of material, we can fulfill this customer requirement. Bye. This will become documented evidence and if you don't have it, Then when you open to the customer, You are not 100% sure that this will fulfill what the customer needs.

You can assume or you can look at the other results that you have which are really close to this Then there is some There is some level of uncertainty and I want to just give you this because this is for example, this is the occurrence and severity. So You have some writing here. It's called severity occurrence and detection and the severity says The severity is measured with the most serious failure effect of a given failure mode.

So basically, if your product fails on something, how severe it is for the customer, the effect of this failure. So you can say very low, So no effect, but let's go for the moderate. Very object-enabled appearance, sound, vibration, harshness or haptics. Haptics means if you touch the part, it's different when you touch it, right? Mm-hmm. Part number five is degradation of secondary vehicle function.

For example, acoustic could be a secondary vehicle function. And you say, okay, if my car in the engine compartment burns, then the performance of the acoustic doesn't absorb the noise that good. So the degradation of the acoustic is there, right? You can imagine if a part is in original state, it has some acoustic performance, but if you burn it, it has only 20% of this acoustic performance. So this is what could be called degradation of secondary vehicle function.

Lots of secondary vehicle functions, so the part will burn totally, and then... degradation of primary vehicle functions. So you can imagine that due to the failure of your part, the primary vehicle function which is that the final user can drive the car is being degraded or lost, right? Then we have the highest, which is non-compliance with regulation, And then health or driver, passengers, road users, pedestrians.

whatever is Wherever the risk of the car user or anywhere in the surrounding pedestrian is at risk, then you need to give the highest number for the severity. Yeah. So you need to write this. You have also done Occurrence, so means What is the likelihood that this will happen? So you need to write that from the extreme high to the high. So you have onlyYeah. Sorry, from the extreme low to the extreme high.

Then you give the numbers of this. And then the last one is Detection. So what are your... What are your methods of detecting this this failure that can occur. How can you detect? How certain you can be? Ability to detect. If you don't have ability to detect, it's very low. If you have ability to detect, it's very high. So for example, if I say If this material is burning with the specific rate, What is my detection?

I have a laboratory method. I can put my material in the laboratory and burn it. I'm burning it. test report available where I have measured my burning speed. So, this you can say, okay, I have... Prove and test method for verification of this function, right? So then the rating is... which gives you the very high detection of your failure. So you minimize the risk, right? And then at the end, the DFMEA you multiply All these three, you multiply the severity number or the...

Before it was numbers, now we operate with low, medium, high. So you have, for example, my severity is high, my occurrence is high and my detection is low. Low means very risky situation because it's very like the effect of this failure severity is very high means something serious. My occurrence is very high, means it can happen quite often. and my detection is very low means I don't have any any means how to detect this failure.

So this is very, very risky situation, right? The most risky combination that you can imagine. So, and then you multiply the software in this, for example, this PLATO software or any other software on the market is multiplying these three factors for the severity, occurrence and detection. and gives you gives you the output if Does it give you, rates your risk? The key behind of multiplying those three is already specified by this IATF.

So here is the table. If you have your severity on the high level, your occurrence on the high level, and your detection on the whatever, because then the severity and occurrence are already so high that you will end up with the H, means high risk. Then you have some cases where your risk is moderate. And then you have some cases when your risk is low, right? For the first, the most important one is the severity.

So you can see even if you have, when you have high severity, For most of those, you will have a high risk. youYeah. Okay, I tried to explain very shortly because this is a very complex tool to assess, but I tried to be quite short. This is how you, coming back to your question, this is how you say what are the pain points in the development that if you have some new materials, Sometimes you are not certain you have higher risk of implementing this material if you have If you know your material, you know how it performs, then it's much easier toto offer with certainty that the performance will be there.

And when you report higher risk, then this customer could reject the offer or reject the material you suggest?

I mean, this DFMA, we perform when we already have the business. Okay. So then it's in the process of development that you identify where your high risks are If you say, okay, my risk is that the car will burn, what I can do? If I don't know how this specific material is burning, so what I need to do is I need to plan my tests to do in the development because then if I make the test, I get the result.

I lower my severity because then I can see, okay, this material is not burning that much. So I need to plan to perform this test. And when I have this result, I can decrease my risk from high, for example, to medium or to low. Yeah.

But then because you already have the business, the costs, you already agreed with the customer. So it's your risk.

Yes, it's our risk. So therefore, even in the RFQ part, our engineers are already, let's say, putting together what we know and what we don't know. This maturity, what I mentioned before, this maturity of the product, how mature it is, how much we know about it. And if the maturity is high, then the development cost will be low because we don't need to perform additional tests to find out how this product will perform.

But if the maturity of this material or combination of material and design is low, then higher costs for sure there will be higher cost in the development because we need to perform more tests and so on and so on right so of course it's the best to offer product that you know with the with the bill of material that you know against the requirements that you already know that maybe you have already something proven in the past.

Yeah. on fire and really high speed So there are this type of new requirements coming with the new product on customer side. And this is where we... We have in the company also teams called pre-development. Teams which will, let's say, Uh... analyze already in advance before we need to quote to the customer they will analyze What are In which direction customer will go in the next few years? What will be the product on the market?

And what this, how our product can contribute to the customer product.

To stay in the market, to stay in the supply chain.

Yes, and for example, our product will need to change from level A to level B in two or three years because the customer will start to... to require from us something totally different because he will switch from combustion engine to battery electric engine, right? So there will be maybe another type of products on the market, right? because you don't need something what you need for combustion engine, you don't need for electric vehicle.

And the other way around, you need new product for electric vehicle. So some products will phase out, some new products will appear, so and then you need to foreseen what customer will require from this new product in the future? And you try to work ahead close to the customer to identify, okay, or the battery cover that we can offer as an autonom, the customer will expect us maybe to mechanically protect the battery, right?

It will also expect to resist against different flammability rate. It can expect that they maybe if there are some gases going from the battery, somehow those gases need to evacuate from our parts. So how we can grant this in the future for our new products and so on. So this is already another stage. This pre-development is to work in advance Maybe you can also include this in your in your software somehow that if you work in advance on something, how then you transfer from pre-development to to this development from one phase of the product to another?

What inputs you have in the pre-development? Do you want to have the same inputs in the development or you want to filter because you already test something and you know that what you analyzed before? Okay, some of First part, part A of this analysis still is needed and brings added value in the development or you analyze something in the pre-development and it's not needed anymore so you don't want to use it.

and you want to focus on something else during the development phase, right? So if you look from the, let's say, bird eye point of view, a bit wider, you have also within the development, you have different phasesYeah. So. This could be also interesting where, and you need to imagine that in different phases of the, the further you go in the development, the more you know. Right? It's like constantly discovering something new and building up your knowledge and then With knowledge on level A, you can develop with this quality, but if you have knowledge on level B, you can develop the product better and better.

And this is like continuous learning and improving. Yeah.

So then, yeah, okay. I understand. It would be nice that more or less that you have in your tools some prospective element looking towards let's say what could be scenarios for two or three years.

Yeah, I would say for sure, yes. And of course, you know, I'm trying to explain this to you on the very basic level to keep it simple, but you can imagine also customer has some level of uncertainty what they will have on the market on or which way they go and sometimes they are considering also five different scenarios. Yeah. So I think for the software it would be good to have also a lot of version scenarios and so on to be quite flexible because then When you evolve during the development, at the beginning, maybe you have five scenarios, then you do some research and you see, okay, those two doesn't make sense anymore.

There are only those three, but okay, after you gain knowledge and know-how, maybe you identify, ah, okay, you connect two dots and you say, okay, maybe this, we should go totally new way or connect solution A with solution B, which will be solution C, right? So I think in this modern development, which is constantly speeding up and the level of The speed of development is constantly increasing. I think being kind of agile and flexible is one of the most important things.

You don't want to... be so structured that you cannot switch from concept A to B and Yeah. Yeah.

No, I'm absolutely sure my colleague will love to hear that. Moes will love to hear that you have to be more agile. That's his new religion. -Definitely, it's very interesting. I have lots of questions, but I don't know if I can ask a few.

Yeah, go ahead.

Because you have mentioned the pre-development phase, would you imagine this to be something similar to the development phase, but something that is being run by automated software that is giving different scenarios as you said. Um... So my question is, about the the FMEA guideline or handbook, you already use plateau. It seems to me that it's really mythological It's also a very tedious process. So you use a plateau for some calculations, but Do you...

Do you do a lot of manual work in it or is there some level of automation that you guys have internally or use third-party software or you think this is no use for?

No, for the moment we do it manually but right now we are connecting this software with another software which there is some AI behind and I know In the future, there will be like Your job will evolve from... Bing doing the work from the scratch. to something that You have a database where all your previous job is already uploaded. The AI is analyzing this and you don't start from scratch from the zero, but then the AI gives you already somePeace.

where you just, you don't do the work from the beginning. You say you only kind of review the existing concept which was generated by AI. Mm-hmm. Do you know what I mean?

Yes. What is this software called?

It's a call up, for example, this. You know this already from... Google call out? No, it's not Google.

No, it's a Siemens call up. It's from Siemens, I think, no?

No, no, I think not. It's another company. You can... It's available on the market. It's what I know it's already present in big companies. Would see, Colab would see. So. You can maybe try to look there. This is design engagement. If you Google this type of tools, it's a design engagement tool. And I think this is one of the. the newest thing on the market and I think this will be more and more the direction.

And it seems to me to also like when, I mean, you touched a little bit on the cost topic that, you know, And also, this is not only on the cost topic, but it's also related to the DFMA methodology. But there's also a lot of dependencies between different factors when you have to make decision. Like if you want to say, okay, we will have this plan for this cost and we have another plan. And this plant is based on, we're going to source our material from here, we're going to manufacture in this plant, etc.

So there's a lot of dependencies between those components. And do you also do this manually or do you use some automation for it? Because I feel like it could be also very complicated to put all those things together and...

examine all those different possibilities. Yes, and also it's also changing. I don't want to go too much in details how we work there because it could be everything which is money related. It's quite confidential. No, just high level. Yeah, but I think from... I would see a potential at least to to switch from manual work to being able to work on the databases or that you give your inputs, your prompts, and it will give you already some, some solution behind, right?

or based on If you have a database where all your previous Part cost calculations are in and then maybe they could analyze and would say, okay, for the similar type of requirement, you propose this type of solution. solution in the past with the lowest cost. So, you know, kind of tool that will analyze the work that you did before already and will try to give you a starting point. Or some suggestion that you can say, okay, yes, I did this in the past.

Because what you need to also imagine for big companies, you have not one engineer who is doing the same job. multiple engineers and they don't know sometimes they don't know what the other engineer has done what he has tested what scenario he has already gone through sometimes they know sometimes they don't So if you have a database and kind of AI engagement that is able to analyze you know, on the larger sample what was already done.

And then it can bring a suggestion to the to the person who needs to do it, for the new one, right?

If you know what I mean. Yeah. And that's the potential, but you don't see this as a burning point at the moment, I guess.

No, not the burning point. I mean, burning point is that you need to be fast. When the customer wants to have the quote to be able to decide, you need to be fast. And if you have multiple scenarios, multiple variants, and multiple parts in managing it to do five scenarios for 10 parts. It's already 15 variants of your the way how you want to make the part, right? And to prepare by one person from the scratch, In short time, it's challenging, right?

Yes, and this occurs often or no?

Because this is what I was imagining.

It's a cumbersome process due to those dependencies and permutations between different... Um... parts of the course, okay.

And, you know, combining this with what I said before, if your product is mature and you know what you have and how how you want to develop, it's easier. But if you work on something new and the maturity level of the solution or bill of material, it's not high,Then, You know, it's more and more difficult to create all those scenarios for all those parts, right?

And the maturity level is also related to another burning point you talked about, which is like if the customer comes to the new requirement, then we're not going to have a high maturity level. We need to do this pre-development phase, you know, see what the customer is about.

This takes time, right? So then if you need to be really fast, it's challenging then.

My question here, would you imagine some simulation and prediction in that phase tools that could be useful? Or this is just a time. Would be nice.

Would be great. To have, you know, some like a golden ball, like a glass ball, which you will tell, okay, how this will... how this will perform or Yeah. But, you know, it's automotive, it's IATF, so you need to have documented information, means you need to have evidences for everything that you do. And you can imagine if the solution that you propose will not work, the customer can come back to you and then say, You promised me this and now this is not working.

I have some failure in the car. I have some malfunction and I have some market returns. So it's then. in this automotive world, it can end up with really thousands of Euro being paid from one side to another, right?

Yeah, I mean, the question is, You can, for example,Hmm. pre-predict before going into this, okay, this material would probably wouldn't work because of certain kind of knowledge or dependency that would prevent it from working. So this would also say,Cost and time.

Yes, I would say the direction to go is to have the tools that can already analyze what you have. Based on the existing results supported with AI can give you the guidance which way to go because one person cannot analyze so many results. But the bigger and bigger your database will be, the better solution you can find, right? Or can be already brought to you by AI or... to start from.

And are you guys doing anything?

I think I need to finish.

Sorry. Thank you.

Very interesting talk, but... I have my tasks to do as well.

Yes, of course. No, definitely. Thank you very much, Marius. Thank you very much for your time. Yeah. Do you have any questions to us?

No, but not at the moment. But it's interesting that what are you trying to do? And I think it's a good direction. Like I mentioned, it's involving this way how we develop parts. It's speeding up. And definitely, these tools in there are needed, right? Yeah. Try to also think, maybe one last comment, try to also think how to get more people,because you have cross functional teams doing the development.

It's not one guy. It's multiple guys. Sometimes it's three, five. Sometimes can be 100 guys. to work on the same topic at the same time. Mm-hmm. Yeah. Definitely. Thank you very much.

Thank you very much for all your insights and your knowledge.

If you find your product, you develop it, and then you are rich and famous, let me know.

We let you know before, no worries.

Okay, guys, good luck. Thank you very much. If you need anything in future, we can do another round. Perfect.

Thank you very much. Thank you. Enjoy your time.

Thanks. Have a good one. Ciao. Sir?

Now we have to get rich and famous. But that was a really good interview. Now we have to get rich and famous. But that was a really good interview. Yeah. Good. Mm.

Ideas: Tool that gives you info about scenarios that had already been testet (Database) —>suggestion to the person who has to do this today

—>Burning point: you have to be fast, if you have multiple scenarios! Very challenging when you start from the scratch

—>esp if working on sth new and if maturity level is not high it is very challenging

—>pre-development phase…will take time

—>you need to have documented evidence for everything you do

Direction to go: having tools that will already analyse what you have and where to go from that point onwards —>Database

—>how to get more people into it..cross functional teams working on the same topic on the same time